Wolverine Snap-14MP Quick Start Guide

The Wolverine SNaP-14MP is a film, photo and slide scanner. It features a built-in color screen and memory card reader to digitize your media without the use of a computer.

Note:

- SNaP-14MP is compatible with any capacity SD/SDHC type of memory cards.

- Improper insertion or removal of memory cards may damage the card or card reader.

- Never Insert or Remove a SD card while the SNaP-14MP unit is on.

Function Keys

- Power Key: Used to turn power on and off.

- Left or Right arrow keys: Used to change the menu on Viewer; to flip or mirror images in Capture mode

- “OK” to activate each menu and make selection.

- “Menu” key: to go back to the home menu; change a mode.

- “Convert” key: to convert film or photo into a digital image

Quick Start Guide

- Attach Power

- Insert a SD/SDHC Memory Card, it plugs into the back of the unit.

- Always power SNaP-14MP off before you remove the memory card.

- Turn SNaP-14MP Digital Converter on.

- Convert Photos:

  - Move the lever to “Photo”, press “OK”

  - Press ARROW keys to select photo size, press “OK”.

  - Open top lid, place in proper Photo Guide.

  - Place photo against Photo Guide, close lid.

  - Optional step - edit the brightness or orientation

  - Press “CONVERT”.

  - Press “OK” to save the image

  - Open the lid, replace photo, repeat steps d – g.

- Convert film or slide:

  - Move the lever “Film”, press “OK”

  - Press ARROW keys to select film type, press “OK”.

  - Place film into proper tray; insert tray into right side slot.

  - Optional step - edit the brightness or orientation.

  - Press “Convert”.

  - Press “OK” to save image.

  - Push film tray to the left for a new picture and repeat steps d – f.

- Turn off the SNaP-14MP

- Eject the SC card.

If you are using the South Central Library System provided SD card please reformat it when you are finished to protect your privacy.

To reformat the SD card insert it into the laptops SD card reader, it’s on the left hand side of the laptop. An autoplay window will appear on the screen. Select “Open folder to view files”. A new window will appear on the screen, right click on “Canon_DC” on the left hand side of the window. If no other devices are plugged into the laptop this should be assigned drive “E”. Select “format” and click “Start” and “OK” then “OK” again when the format is complete. This will erase all the data currently on the SD card.  Remove the SD card from the laptop and place it back in the Wolverine SNaP 14-MP.